Documentation of running DP with “projection” on unitree go2 robot

This is a wrap up of the pipeline implemented by Fuxiao Liao on June 2025.  由于我的项目经历比较少，所以很多代码和文档都很不规范，请多指正，这样我也可以学到很多，感谢

- 廖拂晓

Part 1: Outline

There are three related repositories

- Repo named GADP_quadruped: github branch lowdim, branch lowdim, not in the main branch

  - This repo include training, inference

  - The submodule is the branch lowdim of github branch lowdim, which is the modified Diffusion Policy 3D (Deep learning module)

  - You do not need to read README_GADP_legged.md.  I have included the needed information in this word document.

- Repo named unitree_ros2: github

  - This repo is based on the official repo offered by unitree for ros2 communication

  - I guess there should be one such communication SDK for the unitree humanoid robot G1

  - You do not need to read the english part at the beginning README_zh.md.  I have included the needed information in this word document.

- Repo named legged_robot_IL: github

  - This repo contains one useful script for collecting data

- We use the same python venv and ROS2 foxy for the whole pipeline. Installation following this dp3_install.md is sufficient.

Part 2: Data Collection

- Set up unitree workspace, you should follow the chinese part of README_zh.md.

- For our Imitation Learning framework, the input are the joint angles of the 12 joints in the current time step, the outputs are the velocity on the center of mass of the next time step (in the future potentially the joint angles of the next time step). After reading the unitree documentation website ros2_service, basic_services, sports_services, we got informed of the needed ros2 topics (The according docu of G1 robots might be these ones G1_docu_底层, G1 docu_高层控制 ). Joint states are called /lf/lowstate/low_m{i}q, i from 0 to 12 corresponds to 12 joints. Actions are /lf/sportmodestate/sport_vel{i}, i from 0 to 2, and /lf/sportmodestate/sport_yaw_speed.

- Fuxiao uses all_topics_script to record all the ros2 topics of go2 into csv file when collecting data. For G1 robot, I guess you can ros2 topic list all topics and then feed the topic list and my all_topics_scripts into ChatGPT and ask for a new script. For example you can collect 30 trajectories of c.a. 5 meter resulting into 30 csv files. The next step is to process the 30 csv files into 1 zarr files using csv2zarr script. It need to be emphasize that the 30 trajectories are actually concatenated into one Numpy arrays of “state” and “action”. To preserve the boundaries of the original episodes, a metadata array named `episode_ends` is generated. Detail can be seen in the comment on the top of this script.

- The expert demonstration is let the robot dog move in straight line with constant velocity. This can be done by

  - Either human remote control, but this demand the human operator to be experienced enough to control the robot to move in quasi-constant speed.

  - The unitree go2 offers the user to directly send command of the velocity on the center of mass using the “sport_client_.Move(req_, vx_, vy_, vyaw_)” function. My implementation was vel_cmd_pub.cpp to publish a constant speed and velocity_follower.cpp to subscribe to the velocity command and publish to sport request API.

- With the zarr file, we are ready to start the Training.

Part 3: Training

- In repo GADP_quadruped: github branch lowdim, branch lowdim. Also download the submodule github branch lowdim and checkout to branch lowdim.

- Write a task-relevant config e.g. called G1_walk.yaml and follow these steps

- ## Fuxiao train dp3 on unitree go2 robot

- ```bash

- bash scripts/train_policy.sh simple_lowdim_dp quadruped_walk CMVasACTION 0 0

- #                              policy_name      task_name    additional_info seed gpu_id

- ```

- - This is how we start the training, the comments explain the 5 arguments.

- - The policy_name "simple_lowdim_dp" defines which policy to use. In this case, it will be /GADP_quadruped/3D-Diffusion-Policy/3D-Diffusion-Policy/diffusion_policy_3d/policy/diffusion_unet_lowdim_policy.py and its accroding config file /GADP_quadruped/3D-Diffusion-Policy/3D-Diffusion-Policy/diffusion_policy_3d/config/simple_lowdim_dp.yaml.

- - The task_name "quadruped_walk" corresponds to the config file /GADP_quadruped/3D-Diffusion-Policy/3D-Diffusion-Policy/diffusion_policy_3d/config/task/quadruped_walk.yaml. Here you define the dimension of states and action.And also defines to use the /GADP_quadruped/3D-Diffusion-Policy/3D-Diffusion-Policy/diffusion_policy_3d/dataset/realdex_dataset_state.py for data processing.

- - The additional_info will change the name of the current run in wandb, “loss” , “bc_loss”

3. One critical and tricky thing is the data format of the training data. Basically, it will be
data = {

'obs': agent_pos = sample['state'][:,].astype(np.float32)

, # T, D_obs

'action': sample['action'].astype(np.float32) # T, D_action

}

The zarr file will be processed by /GADP_quadruped/3D-Diffusion-Policy/3D-Diffusion-Policy/diffusion_policy_3d/dataset/realdex_dataset_state.py . 


Part 4: Inference

- The folder action_infer_legged in repo GADP_quadruped is used for inference. Script action_infer_quadruped_3.py is for infer trained dp3 model without the projection mechanism. Action_infer_quadruped_proj.py is for infer the trained dp3 model, together with projection mechanism.

- The details of setting up the ros2 workspace are

conda deactivate

source /opt/ros/foxy/setup.bash

conda activate dp3_legged

cd /home/guest/GADP_ros2/GADP_quadruped/action_infer_legged/ros2_ws

colcon build

source install/setup.bash

# for inference without projection mechanism

ros2 run action_infer_legged action_infer_quadruped_3

# for inference with projection mechanism

ros2 run action_infer_legged action_infer_quadruped_proj

- To test the unitree module, we can try to give constant x-directional velocity on unitree API. velocity command publisher script

source /opt/ros/foxy/setup.bash

cd unitree_ros2/

source ~/unitree_ros2/setup.sh

- To run the inference on robot

# publish the motor state such that the dp3 can subscribe

ros2 run unitree_ros2_example motor_state_publisher

# subscribe the output of dp3 model and publish the output as the velocity command to

# unitree api

ros2 run unitree_ros2_example velocity_follower

Part 5: G1 specific information

- G1_docu_底层.  The motor angle is very likely in “/rt/lowstate “, similar to go2, the values could be “low_m{i}_q} in /rt/lowstate.

- The screenshot from G1 docu_高层控制. The velocity command on the center of mass might use this function offered by unitree for G1 robot. Fuxiao guesses this because the Move function is used for controlling the go2 robot dog in fuxiao’s current pipeline.